Pilot's

Operating

Handbook

Van's Aircraft RV-7

PH-XXX

© 202 by Author

Document History

| Date | Revision | Description | Author |
| --- | --- | --- | --- |
| 2022-10-10 | 1.0 | Initial setup with Van's performance data | Author |

Type		Van's Aircraft

Model		RV-7

Serial No.	74612

Engine		Superior IO360-M1B

Propeller	MT MTV-12B

Built		September 2017 – October 2022

Builder		Name

NVAV No.	Number

The airplane must be operated in compliance with information and limitations described in this document.

Disclaimer

The Van's Aircraft RV-7 is classified as experimental, it has not been certified and does not comply with international safety regulations.

Flight characteristics such as high roll rate and light control forces can be more demanding to the pilots’ skill.

This airplane has been built by the builder to the best of his knowledge and quality of craftsmanship. Nonetheless, operating this airplane is at the sole risk of the pilot.

Table of Contents

Introduction	1

Specifications	3

Performance	4

Limitations	7

Aerobatic Maneuvers	9

Weight & Balance	11

Limitations	11

Center of Gravity Range	13

Loading Form	14

Loading Scenarios	14

Operating Instructions	17

Preflight	17

Startup	20

Takeoff	21

Cruise	22

Landing	22

Shutdown	23

Securing	23

Emergency Procedures	25

Electrical Failure	25

Engine Failure	26

Fires	27

Definitions & Abbreviations	29

Units & Conversions	34

Tables	34

Notes	36

Compass Headings	36

ATC Light Signals	36

# Introduction

PH-YAR is a Van’s Aircraft RV-7, high performance, single engine, low wing, side by side two seat, all-aluminum, tail-dragger airplane. It is an all-around experimental sport airplane, with excellent cross-country capability, fine aerobatic qualities, superior handling and capable of taking off and landing in virtually any airport. It was built from a Van’s standard kit by Joop Sint Jago during 2017 until 2022.

This Pilot's Operating Handbook contains information and limitations for the safe and efficient operation of this experimental airplane. This airplane must be operated by minimum one crew. However, the aircraft may be operated from either the right or left seat. Controls are fully redundant. For convenience the right control stick is removable.

Provided that national operational requirements are met, the approved operation is VFR, IFR and Night. This aircraft has no oxygen system installed. For extended flights above 3 km (10'000 ft) AMSL, a portable oxygen system is recommended.

Placards

# Specifications

# Performance

Solo Weight

Aerobatic Weight

Gross Weight

Stalling Speeds

Cruise Performance

Glide Performance

# Limitations

Operation of the airplane outside of the specified operating limitations is not permitted.

Airspeeds

Airspeed Indicator Markings

Pay special attention to the maneuvering speed. As it is a function of stall speed in clean configuration, it is relatively low compared to the cruising and top speeds of this airplane. Above this speed, maximum elevator deflection can induce loads way beyond the design limit. At VNE , max elevator would result in loads > 10g’s!

Fuel

## Aerobatic Maneuvers

Load Factors

# Weight & Balance

Datum

## Limitations

Determining empty CG

SI Units

## Loading Form

# Systems Description

# Operating Instructions

## Preflight

Preflight continued

Preflight continued

## Startup

## Takeoff

## Cruise

## Landing

## Shutdown

## Securing

# Emergency Procedures

## Electrical Failure

## Engine Failure

## Fires

Fires continued

# Definitions & Abbreviations

# Units & Conversions

## Tables

# Notes

## Compass Headings

## ATC Light Signals

Mass is the correct term, although weight is (wrongfully) used by many people